###### Ventilator Troubleshooting Alarms

Skills Checklist

Child’s Name: _______________________________

Person Trained: _____________________________ Position: _____________________ Instructor: _____________________

|  | Demo | Return Demonstration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Date | Date | Date | Date | Date | Date | Date |
| A. States name and purpose of procedure. |  |  |  |  |  |  |  |
| B. Steps: 1. Identifies which alarm is sounding |  |  |  |  |  |  |  |
| 2. Checks child first, if LOW PRESSURE alarm sounds |  |  |  |  |  |  |  |
| - Removes child from ventilator and gives breaths with resuscitator bag |  |  |  |  |  |  |  |
| 4. Checks for leaks; if child is fine: |  |  |  |  |  |  |  |
| - Child disconnected |  |  |  |  |  |  |  |
| - Disconnected tubing |  |  |  |  |  |  |  |
| - Kinked tubing |  |  |  |  |  |  |  |
| - Punctured tubing |  |  |  |  |  |  |  |
| - Water in exhalation valve |  |  |  |  |  |  |  |
| - Hole in exhalation valve |  |  |  |  |  |  |  |
| - Loose fitting heater humidifier |  |  |  |  |  |  |  |
| - Accidental change in ventilator settings |  |  |  |  |  |  |  |
| 5. Tests system after leak is found |  |  |  |  |  |  |  |
| 6. Places child back on ventilator |  |  |  |  |  |  |  |
| 7. Checks child first if a HIGH PRESSURE alarm sounds |  |  |  |  |  |  |  |
| 8. Checks activity of child: |  |  |  |  |  |  |  |
| - Coughing |  |  |  |  |  |  |  |
| - Sneezing |  |  |  |  |  |  |  |
| - Talking |  |  |  |  |  |  |  |
| - Laughing |  |  |  |  |  |  |  |
| - Crying |  |  |  |  |  |  |  |
| - Hiccups |  |  |  |  |  |  |  |
| - Body position |  |  |  |  |  |  |  |
| - Breath holding |  |  |  |  |  |  |  |
| 9. Suctions, if needed |  |  |  |  |  |  |  |
| 10. Changes trach tube, if needed |  |  |  |  |  |  |  |
| 11. Removes child from ventilator and gives breaths with resuscitator bag |  |  |  |  |  |  |  |
| 12. Checks ventilator for obstructions, if child is okay: |  |  |  |  |  |  |  |
| - Kinks in tubing |  |  |  |  |  |  |  |
| - Water in tubing |  |  |  |  |  |  |  |
| - Blocked exhalation valve |  |  |  |  |  |  |  |
| - Accidental change in ventilator settings |  |  |  |  |  |  |  |

###### Continued next page 
Ventilator Troubleshooting Alarms

Skills Checklist/page 2

Child’s Name: _______________________________

| 13. Places child back on ventilator once problem is solved |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 14. Checks below if POWER SOURCE alarm: |  |  |  |  |  |  |  |
| - AC power |  |  |  |  |  |  |  |
| - Internal battery |  |  |  |  |  |  |  |
| - External battery |  |  |  |  |  |  |  |
| - Removes child from ventilator if all 3 systems fail and gives breaths with resuscitator bag |  |  |  |  |  |  |  |

Checklist content approved by:

___________________________________________________		_________________________________

Parent/Guardian Signature				Date